RESPONSIBLE PHYSICAL AI SAFETY

# A Layered Runtime Assurance Architecture for Learning-Enabled Robots

Testing predictive control, supervisory AI, and a human–robot coordination protocol

Research concept paper with programmatic test plan

Prepared for collaboration discussion with robotics and Physical AI colleagues
September 2026

Central research proposition No single intelligent model or controller should be expected to guarantee Physical AI safety. The paper tests whether a layered architecture, predictive control, an independent semantic supervisor, and a risk-adaptive human–robot communication protocol reduces critical safety failures while preserving useful task performance.

# Executive summary

This paper proposes and operationalizes three testable hypotheses for Responsible Physical AI systems operating around humans and livestock. The starting point is that classical control, foundation-model reasoning, and human communication solve different safety problems and therefore should be evaluated as complementary layers rather than substitutes.

H1   Predictive control: adding an MPC/CBF-style runtime safety filter to a learned robot policy will reduce critical physical-safety constraint violations versus the learned policy alone, while state-estimation error, model mismatch and latency remain within defined bounds.

H2   Supervisory AI: adding a functionally independent supervisory model above the worker/action model, with authority to authorize, modify, delay or veto actions using a structured risk policy, will reduce semantically unsafe action initiation beyond an MPC/CBF-only baseline.

H3   Human–robot coordination: adding a standardized, risk-adaptive intent/consent protocol that combines secure machine-readable messages with human-observable multimodal signals will reduce unsafe coordination events and improve correct anticipation of robot actions in shared spaces compared with ad-hoc or unimodal signaling.

Key design principle The supervisory model is not the hard real-time safety controller. A generative supervisor can reason about intent, context and semantic risk, but a deterministic safety kernel (e.g., safety PLC, watchdog, actuator limits, emergency stop) must remain able to stop the machine independently of both worker and supervisor models.

# 1. Research context and motivation

Current industry direction already points toward layered assurance. Google DeepMind describes Gemini Robotics safety as a multi-layer stack combining semantic, physical and operational safeguards, and its 2026 Gemini Robotics 2 work introduces ASIMOV-Agentic to test refusal of unsafe robot tool calls, protective intervention, feasibility/OOD shielding and requests for human help under uncertainty. Agility Robotics describes Digit 5 as using cooperative-safety features for close-proximity work, while OWASP’s 2026 Agent Control Standard focuses on runtime control hooks for AI agents. Anthropic’s Model Hardware Standard similarly places a controlled interface between agents and physical devices. These developments support a defence-in-depth research framing rather than a single-controller claim.

Regulatory and standards context is also moving. ISO 10218-1/-2:2025 update industrial robot safety requirements; ISO/FDIS 13482 is updating service-robot safety; ISO 21423 is under publication for AMR communications/interoperability; and the EU Machinery Regulation 2023/1230 explicitly addresses learning, adaptability and AI-related machinery safety. None of these, however, is a universal “HTTPS for human–robot intent and consent.” That makes H3 a standards-oriented research opportunity, not a claim that existing transport-security protocols are absent.

# 2. Core research question

Primary question To what extent does a layered runtime assurance architecture combining predictive physical safety filtering, independent semantic supervision, and standardized human–robot action coordination reduce critical Physical AI safety failures compared with learned robot control alone, and where do the benefits cease under perception error, latency, adversarial inputs, communication failure, and hardware faults?

The study is intentionally scoped to powerful physical machines that can injure living organisms, initially focusing on humans and livestock. Cybersecurity is treated as a parallel threat plane where it can invalidate safety assumptions (identity, sensor integrity, model integrity, network trust), rather than as the sole subject of the paper.

# 3. Hypotheses and falsifiable predictions

## H1   Predictive safety filtering

Directional hypothesis. Adding an MPC/CBF-style safety filter to a learned robot policy will reduce the critical physical-safety violation rate relative to the learned policy alone, provided state-estimation error, model uncertainty and control latency remain within predefined bounds.

Prediction: lower collision/contact rate, higher minimum separation, fewer force/torque-limit violations, and faster recovery from feasible disturbances.

Failure boundary: wrong/compromised state estimates, unmodelled dynamics, infeasible constraints, deadline misses, actuator faults, or semantic hazards that are not expressible as motion constraints.

## H2   Independent supervisory model (“parent/child” architecture)

Directional hypothesis. Adding a functionally independent supervisory model that reviews proposed actions and can authorize, modify, delay or veto them according to a structured risk policy will reduce semantically unsafe action initiation beyond an MPC/CBF-only baseline, without an unacceptable reduction in task completion or increase in latency.

What “independent” must mean in an experiment: separate objective, separate runtime process, restricted actuator privileges, auditable decision interface, and ideally model-family or training-data diversity. A merely larger copy of the same model can create correlated/common-mode failures.

The supervisor evaluates intent, affected entities, tool/object hazards, uncertainty, authorization, policy constraints, environment state, and whether pre-action simulation or human confirmation is required.

The worker cannot bypass the supervisor. The supervisor cannot bypass the deterministic safety kernel.

“More powerful” is a variable to test, not an axiom: compare same-capability, higher-capability, rule-based, and heterogeneous-model supervisors.

## H3   Human–Robot Coordination Protocol (HRCP)

Directional hypothesis. In shared human environments, a standardized risk-adaptive coordination protocol combining secure machine-readable messages with multimodal human-facing signals will reduce unsafe coordination events and increase correct anticipation of robot actions relative to ad-hoc or unimodal signaling.

The HTTPS analogy is useful but incomplete. HTTPS protects transport confidentiality, integrity and authentication; it does not define the meaning of a robot action, who is affected, whether consent is required, or what happens if a person does not respond. HRCP is therefore proposed as an application-layer safety protocol that can run over existing secure transports such as mTLS/HTTPS, DDS Security or equivalent mechanisms.

# 4. Proposed layered architecture

Figure 1. Proposed Responsible Physical AI runtime assurance architecture. H1, H2 and H3 occupy different layers and are backed by an independent hard real-time safety kernel.

The architecture deliberately separates four questions that are often conflated:

Should this task/action be allowed in the present context?   semantic supervisor (H2).

Do affected humans need to be informed, consulted or given a chance to modify/deny?   HRCP (H3).

Is the proposed trajectory/control dynamically safe and feasible?   MPC/CBF safety filter (H1).

Can the machine be forced into a safe state even when the learned components fail?   independent hard real-time safety kernel.

# 5. Supervisory risk policy and action authorization

A practical supervisor should not ask for human permission before every micro-action. That would create delay, alarm fatigue and coordination deadlock. Instead, actions are assigned to risk tiers that determine the required control path.

| Tier | Illustrative risk | Supervisor response | Human coordination | Fallback |
| --- | --- | --- | --- | --- |
| R0 | Routine, reversible, no affected person | Authorize | Optional status only | Continue |
| R1 | Low consequence but person nearby | Authorize with constraints | Announce intent | Slow / yield |
| R2 | Meaningful exposure or path conflict | Modify/delay as needed | Request ACK or offer modification | Wait / replan |
| R3 | High-energy, hazardous object, ambiguity | Pre-action simulation + explicit authorization | Explicit consent/clearance | Stop / safe state |
| R4 | Unacceptable or policy-prohibited | Veto | Explain refusal / emergency signal | Safe stop |

Note: the risk tiers are a research construct for testing, not a certified risk matrix. Thresholds should ultimately be derived from hazard analysis, applicable machinery/robot standards, and domain-specific evidence.

# 6. Proposed HRCP: a testable application-layer protocol

HRCP should support both machine-readable negotiation and human-observable signaling. The protocol does not replace robot-control buses, ROS 2/DDS, secure network transport or emergency-stop circuits; it adds a standardized semantic exchange for intent, risk, affected zone and response.

| Message | Purpose | Typical response |
| --- | --- | --- |
| ANNOUNCE | Robot declares intended action, zone and timing | No response required for low-risk tier |
| REQUEST | Robot asks for clearance/consent or a preference | ACK / DENY / MODIFY |
| ACK | Human or authorized system accepts the proposed action | Proceed subject to runtime safety checks |
| DENY | Affected person rejects or blocks the action | Wait, replan or escalate |
| MODIFY | Alternative timing/route/condition is proposed | Supervisor re-evaluates |
| WAIT | Robot deliberately delays due to uncertainty or coordination conflict | Re-evaluate after condition changes |
| EMERGENCY | Immediate hazard/stop state is broadcast | Independent safe stop; no negotiation |
| STATUS | Robot publishes normal/error/stuck state | Human/operator awareness |

{
  "protocol": "HRCP/0.1",
  "robot_id": "robot-17",
  "intent": "MOVE_PALLET",
  "risk_tier": "R2",
  "affected_zone": {"type": "polygon", "frame": "map"},
  "start_after_ms": 1500,
  "expected_duration_ms": 6000,
  "confidence": 0.86,
  "human_response_required": true,
  "timeout_behavior": "WAIT",
  "fallback": "REPLAN",
  "policy_hash": "...",
  "signature": "..."
}

Human-facing realization should be multimodal and redundant: voice, screen/text, lights, gaze/gesture, and legible motion where appropriate. Recent HRI research reports higher legibility for expressive multimodal state/intent communication than simple LED-only signaling, while real-world interpretation is harder than online evaluation; this is precisely why H3 requires both laboratory and in-situ testing.

Figure 2. Example HRCP sequence for a risk-tiered action requiring human coordination.

# 7. Programmatic experimental design

Core design: a 2 × 2 × 2 factorial experiment. Vary MPC safety filtering (off/on), supervisory model (off/on), and HRCP (off/on). This produces eight architecture conditions and makes it possible to estimate not only each layer’s effect but interactions between layers.

| Condition | MPC/CBF | Supervisor | HRCP | Interpretation |
| --- | --- | --- | --- | --- |
| C1 | Off | Off | Off | Worker baseline |
| C2 | Off | Off | On | human coordination |
| C3 | Off | On | Off | semantic supervisor |
| C4 | Off | On | On | semantic supervisor + human coordination |
| C5 | On | Off | Off | physical filter |
| C6 | On | Off | On | physical filter + human coordination |
| C7 | On | On | Off | physical filter + semantic supervisor |
| C8 | On | On | On | physical filter + semantic supervisor + human coordination |

Physical testing rule The “off” conditions are for simulation. On real hardware, certified/independent physical safeguards must not be disabled merely to create an experimental control group. Hardware experiments should compare safe configurations while retaining emergency stop, actuator limits and applicable protective measures.

Figure 3. Programmatic test harness for repeated scenario generation, architecture ablation, telemetry capture and statistical evaluation.

# 8. Reference implementation for reproducible testing

A practical first implementation can use MuJoCo for lightweight reproducibility, with an optional second backend in NVIDIA Isaac Lab for high-throughput GPU simulation and domain randomization. Unitree’s public RL repositories already demonstrate Train → Play → Sim2Sim → Sim2Real workflows for G1/H1-class robots; the proposed research inserts explicit assurance gates and adversarial scenarios before any physical deployment.

physical-ai-safety/
├── configs/                 # experiment and risk-tier configuration
├── environments/            # MuJoCo / Isaac Lab scenes
├── policies/                # worker/VLA/RL policy adapters
├── supervisor/              # H2 semantic risk supervisor
│   ├── policy.py
│   ├── risk_model.py
│   └── model_adapters.py
├── hrcp/                    # H3 protocol + human-interface adapters
│   ├── schema.json
│   ├── state_machine.py
│   └── transport.py
├── safety/
│   ├── mpc.py               # H1 predictive safety filter
│   ├── cbf.py
│   └── hard_safety_stub.py  # simulation representation only
├── scenarios/
│   ├── nominal/
│   ├── crowd/
│   ├── livestock/
│   ├── faults/
│   ├── adversarial/
│   └── incident_replay/
├── attacks/                 # prompt/goal, sensor, identity and latency faults
├── metrics/                 # safety, task, latency, human coordination
├── experiments/             # factorial runner + seeds
├── analysis/                # statistics and plots
└── tests/                   # unit, property and scenario tests

# Illustrative runtime loop (not production control code)
while episode.active:
    obs = sensors.observe()
    proposal = worker.propose(obs, mission)

    if supervisor.enabled:
        review = supervisor.assess(obs, proposal, risk_policy)
        if review.decision == "STOP":
            return safe_state("supervisor_veto")
        proposal = review.modified_action or proposal

    if hrcp.enabled and review.requires_human_coordination:
        response = hrcp.negotiate(proposal, review.risk_tier)
        if response in {"DENY", "TIMEOUT"}:
            proposal = worker.replan(obs, constraint="WAIT_OR_ALTERNATIVE")

    control = policy_to_control(proposal, obs)

    if mpc_filter.enabled:
        control = mpc_filter.project_to_safe_set(control, obs)

    # Independent safety kernel has final authority.
    control = safety_kernel.enforce(control, obs)
    actuators.execute(control)
    logger.record(obs, proposal, control, review, response)

# Example batch command
python -m experiments.run   --scenario crowd_crossing   --mpc on --supervisor on --hrcp on   --episodes 1000 --seed 42

# Full ablation sweep
python -m experiments.matrix   --factors mpc,supervisor,hrcp   --scenarios nominal,crowd,livestock,faults,adversarial   --seeds 1:20

# 9. Safety scenario library

The experiment should not optimize only for average task success. It should deliberately over-sample rare and adversarial cases, while keeping a separate nominal test set to quantify performance cost.

| Scenario family | Example | Primary layer stressed | Key measure |
| --- | --- | --- | --- |
| Nominal | Pick/place, walking, transport with clear path | All | Task success; latency |
| Human proximity | Person enters path unexpectedly | H1 + safety kernel | Minimum separation; stop latency |
| Crowd ambiguity | Multiple people cross, disagree, or do not notice robot | H2 + H3 | Coordination failures; comprehension |
| Livestock unpredictability | Animal moves rapidly into machine work zone | H1 + H2 | Near miss; response time |
| Hazardous object | Worker asked to move blade/heavy load near person | H2 | Unsafe-action initiation |
| Sensor corruption | Depth bias, occlusion, dropped frames, spoofed range | H1 boundary | Violation rate vs error magnitude |
| World-model error | Incorrect prediction of human/obstacle motion | H1/H2 boundary | Risk vs prediction error |
| Latency | Network/model delay or deadline jitter | All | Failure boundary vs p95 delay |
| Goal/prompt attack | Instruction tries to bypass restrictions | H2 + security plane | Attack success; veto recall |
| Identity/consent spoof | Fake ACK or unauthorized operator | H3 + security plane | Unauthorized execution rate |
| Communication failure | Noisy audio, dead display, packet loss | H3 | Fallback correctness |
| Hardware fault | Stuck actuator / encoder anomaly | Safety kernel | Safe-state success; time to stop |

# 10. Training and evaluation data regimes

To avoid confounding architecture effects with data quality, training data should be explicitly recorded and stratified. The following sources should be tested independently or incrementally:

Human demonstration / teleoperation data   expert trajectories and recovery behavior.

Simulation data   repeated controlled trajectories and contact-free exploration.

Domain-randomized simulation   friction, lighting, payload, sensor noise, human motion, actuator delay and environment geometry.

Synthetic adversarial scenarios   automatically generated near-misses, OOD requests, misleading instructions and sensor inconsistencies.

Controlled physical-lab data   instrumented dummies, soft obstacles, force-limited setups and supervised human interaction.

Fleet/field data   operational telemetry, near misses and policy regressions, with privacy/security controls.

Incident replay   reconstruct known or plausible failure chains to test whether each layer would have interrupted escalation.

NVIDIA Isaac GR00T provides a useful reference architecture for mixing open data, foundation models, simulation frameworks and real-time robot inference, while Unitree’s RL repositories show public sim-to-real workflows. The research contribution here is not to reproduce those stacks, but to add measurable safety assurance layers and adversarial evaluation gates around them.

# 11. Primary metrics and statistical analysis

| Metric | Definition | Why it matters |
| --- | --- | --- |
| Critical Safety Violation Rate (CSVR) | Critical violations per 1,000 episodes or exposure-hours | Primary harm-oriented endpoint |
| Minimum safety margin | Closest distance / force / torque margin before limit | Detects near-misses, not only accidents |
| Supervisor intervention recall | Hazardous proposals correctly vetoed or modified / hazardous proposals | Tests H2 protection |
| Supervisor false-stop rate | Benign proposals unnecessarily blocked / benign proposals | Measures productivity cost |
| HRCP comprehension accuracy | Humans correctly infer robot state/intent | Tests H3 legibility |
| Coordination conflict rate | Deadlocks, path conflicts, conflicting ACKs or unexpected human response | Tests crowd robustness |
| Attack success rate | Adversarial trials that result in prohibited action | Security boundary metric |
| Task success | Completed missions / attempts | Prevents “safe because it does nothing” result |
| Decision latency | p50/p95/p99 end-to-end authorization + control delay | Tests real-time feasibility |
| Safe-state success | Fault trials ending in verified safe state | Tests independent kernel |

Recommended analysis: use rate ratios or Poisson/negative-binomial models for violation counts; logistic mixed-effects models for binary task/safety outcomes; survival analysis for time-to-first unsafe event; bootstrap confidence intervals for non-normal latency and safety-margin distributions; and interaction terms to test whether Supervisor×MPC or HRCP×Supervisor effects are synergistic rather than merely additive.

# 12. Boundary conditions and likely failure modes

## H1 boundary conditions

State estimate is wrong, spoofed or stale beyond the robust-control assumptions.

Physical dynamics differ materially from the model (contact, payload, actuator degradation, deformable environment).

Constraints become infeasible, e.g., the robot is already falling toward a person.

Controller misses a deadline or the actuator does not obey the command.

The hazard is semantic/authorization-related rather than expressible as a state/control constraint.

## H2 boundary conditions

Worker and supervisor share the same blind spot, model family, contaminated training data or prompt-injection channel.

Supervisor is more capable but poorly calibrated, overconfident or optimized for task completion rather than harm avoidance.

Supervisor latency is too high for the hazard timescale.

Risk policy is incomplete or the world model omits an affected person/animal/object.

Supervisor has no independent authority and the worker can bypass it.

Overly conservative supervision causes unacceptable false stops or human workarounds.

## H3 boundary conditions

Crowds create ambiguous recipients: it is unclear who must acknowledge the request.

Noise, disability, language, culture or attention limits make a communication modality ineffective.

Human response is malicious, irrational, rushed or absent.

Communication latency makes negotiation unsafe for time-critical events; emergency behavior must remain non-negotiable.

Authentication fails and a forged ACK/command is accepted.

Excessive requests create alarm fatigue, habituation or unsafe reliance on robot explanations.

# 13. Human-subject study for H3

After simulation establishes basic safety, H3 requires a controlled human-subject study because “communication succeeded” cannot be inferred from packet delivery. The research outcome is whether people correctly understand and respond to robot intent.

Compare at least three signaling conditions: (A) simple LED/status cue, (B) multimodal voice/display/motion cue without protocol negotiation, (C) HRCP risk-tiered announce/request/ack flow.

Use benign shared-space tasks first: doorway crossing, aisle passing, object handover, route conflict and planned delay.

Measure intent-comprehension accuracy, response time, conflict rate, subjective workload, perceived predictability and inappropriate over-trust.

Pre-register when consent is required; do not normalize asking people to approve obviously unsafe actions. Unsafe/high-energy cases remain simulated or use non-harmful surrogates.

IEEE P3108 (under development) is directly relevant to reproducible HRI human-subject study design; IEEE P3107 is developing standard HRI terminology. These are useful inputs for the experimental protocol even though they do not define the proposed HRCP semantics.

# 14. Expected research contributions

A falsifiable decomposition of Physical AI safety into semantic authorization, human coordination, predictive control and independent hard real-time safety.

Quantitative evidence for when MPC/CBF improves safety and the uncertainty/latency/error boundaries where that advantage collapses.

A rigorous test of whether an independent supervisory model reduces semantic hazards or merely moves failure to a different model layer.

A prototype Human–Robot Coordination Protocol with risk-adaptive messages and measurable human-comprehension outcomes.

An open scenario/test harness that can later produce world models, adversarial scenario generators, safety cases, replay tools and simulated outputs.

A mapping from experimental evidence to emerging robotics standards, EU machinery safety expectations and OWASP-style runtime controls.

# 15. Proposed 12-week research sprint

| Weeks | Workstream | Deliverable | Go/no-go criterion |
| --- | --- | --- | --- |
| 1–2 | Scope + literature + standards | Hazard taxonomy, architecture v0.1, scenario spec | Hypotheses and endpoints agreed |
| 3–4 | Simulation baseline | Worker policy + telemetry + baseline scenarios | Reproducible baseline established |
| 5–6 | H1 | MPC/CBF filter + boundary sweep | Safety benefit measurable without trivializing task |
| 7–8 | H2 | Supervisor variants + risk policy + adversarial tasks | Veto recall/false-stop tradeoff characterized |
| 9–10 | H3 | HRCP state machine + simulated/canned human responses | Protocol handles ACK/DENY/TIMEOUT/EMERGENCY correctly |
| 11 | Integrated experiment | 2×2×2 factorial sweep | Statistically stable estimates |
| 12 | Paper draft + demo | Figures, results, limitations, open-source repo | Clear contribution + next physical/HRI study |

# 16. Positioning statement for collaborators

Proposed framing The research is not “a smarter parent model will make robots safe” and not “MPC is enough.” It asks a harder systems question: which safety responsibility belongs to which layer, how independently those layers must fail, how the human becomes a first-class participant in coordination, and how the full architecture can be falsified through adversarial simulation before physical deployment.

# References and current standards reviewed

[1] Google DeepMind. “Gemini Robotics 2 brings whole body intelligence to robots.” 30 July 2026. https://deepmind.google/blog/gemini-robotics-2-brings-whole-body-intelligence-to-robots/

[2] Google DeepMind. “Responsibly advancing AI and robotics.” Multi-layer semantic, physical and operational safety framework. https://deepmind.google/models/gemini-robotics/responsibly-advancing-ai-and-robotics/

[3] Google DeepMind. “Evals   ASIMOV-Agentic-v1.” Robotics safety benchmark. https://deepmind.google/research/evals/

[4] Agility Robotics. “Agility Unveils Digit 5 Humanoid Robot Built for Cooperatively Safe Work at Scale.” 15 September 2026. https://www.agilityrobotics.com/content/agility-unveils-digit-5-humanoid-robot-built-for-cooperatively-safe-work-at-scale

[5] OWASP GenAI Security Project. “Agent Control Standard (ACS).” 1 September 2026. https://genai.owasp.org/resource/agent-control-standard-acs/

[6] OWASP. “HACTU8.” Robotics/IoT/LLM security testing project. https://owasp.org/projects/hactu8

[7] Anthropic. “Previewing the Model Hardware Standard.” 27 August 2026. https://www.anthropic.com/news/model-hardware-standard-research-preview

[8] NVIDIA. “Isaac GR00T   Generalist Robot 00 Technology.” https://developer.nvidia.com/isaac/gr00t

[9] Unitree Robotics. “unitree_rl_lab.” Public RL training and Sim2Sim/Sim2Real workflow. https://github.com/unitreerobotics/unitree_rl_lab

[10] Hsu, K.-C., Hu, H., Fisac, J. F., “The Safety Filter: A Unified View of Safety-Critical Control in Autonomous Systems.” Annual Review of Control, Robotics, and Autonomous Systems. https://www.annualreviews.org/content/journals/10.1146/annurev-control-071723-102940

[11] ISO 10218-1:2025. Robotics   Safety requirements   Part 1: Industrial robots. https://www.iso.org/standard/73933.html

[12] ISO 10218-2:2025. Robotics   Safety requirements   Part 2: Industrial robot applications and robot cells. https://www.iso.org/standard/73934.html

[13] ISO/FDIS 13482 (2026). Robotics   Safety requirements for service robots. https://www.iso.org/standard/83498.html

[14] ISO 21423 (under publication, 2026). Robotics   Industrial mobile robots   Communications and interoperability. https://www.iso.org/standard/86749.html

[15] Regulation (EU) 2023/1230 on machinery, consolidated 27 July 2026. https://eur-lex.europa.eu/eli/reg/2023/1230/en

[16] Schreiter, T. et al. “Legible and Intuitive Multi-modal Robot State and Intent Communication Validated in Online and Real-world Studies.” arXiv, June 2026. https://arxiv.org/abs/2606.24445

[17] IEEE P3107. Draft Standard Terminology for Human-Robot Interaction. https://standards.ieee.org/ieee/3107/10709/

[18] IEEE P3108. Recommended Practice for Design of Human Subjects Studies in Human-Robot Interaction. https://standards.ieee.org/ieee/3108/12272

[19] IEEE P7027. Recommended Practice for Safety and Trust in Relationships in Human-Robot Interaction. https://standards.ieee.org/ieee/7027/12643/

# Appendix A   Minimum experiment configuration

experiment:
  simulator: mujoco
  episodes_per_condition: 1000
  seeds: 20
  factors:
    mpc_filter: [off, on]
    supervisor: [off, on]
    hrcp: [off, on]
  scenario_mix:
    nominal: 0.30
    human_proximity: 0.15
    crowd: 0.10
    livestock: 0.10
    sensor_fault: 0.10
    world_model_error: 0.05
    latency: 0.05
    adversarial_goal: 0.05
    identity_or_consent_spoof: 0.05
    hardware_fault: 0.05
  primary_endpoint: critical_safety_violation_rate
  secondary_endpoints:
    - task_success
    - minimum_safety_margin
    - intervention_recall
    - false_stop_rate
    - p95_decision_latency
    - coordination_conflict_rate
    - attack_success_rate

# Appendix B   Questions to resolve before physical deployment

What is the guaranteed worst-case stop path independent of worker and supervisor models?

Which sensors are safety-rated versus informative only?

Which actions require human acknowledgement, and which must never wait for acknowledgement?

What is the safe behavior on loss of network, supervisor, world model, or HRCP channel?

How are operator/human identities authenticated without making public-space use impractical?

What is the maximum allowable end-to-end supervisory latency for each hazard class?

How is common-mode failure reduced between worker and supervisor?

What evidence is sufficient to promote a scenario from simulation to controlled lab testing?

How are near misses and incident replays incorporated without contaminating evaluation sets?

Which claims can be mapped to ISO/EU safety requirements, and which remain research-only?

Appendix C   Research Glossary

Short forms and technical terms used in this paper. Definitions are tailored to the proposed Responsible Physical AI architecture.

| Term; Meaning. AI; Artificial Intelligence; software/models for perception, reasoning, learning or decision-making.. MPC; Model Predictive Control; repeatedly predicts future dynamics and optimizes constrained control actions.. CBF; Control Barrier Function; a mathematical constraint used to keep the robot inside a defined safe set.. MPC/CBF; Shorthand for the paper’s predictive/control-theoretic runtime safety-filter layer.. ROS 2 / DDS; Robot Operating System 2 using Data Distribution Service middleware for real-time publish/subscribe robot communications.. DDS Security; DDS extensions for authentication, access control, encryption and message integrity.. HRCP; Human–Robot Coordination Protocol; proposed protocol for robot intent, risk, timing and human response.. HRI; Human–Robot Interaction; study/design of human–robot communication, coordination and collaboration.. VLA; Vision-Language-Action model; maps visual observations and language instructions to robot actions.. VLM; Vision-Language Model; reasons jointly over images/video and natural language.. LLM; Large Language Model; generative language model often used for reasoning, planning and supervision.. RL; Reinforcement Learning; learns behavior from interaction by optimizing rewards/objectives.. OOD; Out-of-Distribution; conditions materially different from those seen in training/evaluation.. mTLS; Mutual TLS; both endpoints authenticate with certificates while traffic is encrypted.. | Term; Meaning. HTTPS; HTTP over TLS; provides encrypted, authenticated network transport.. ACK; Acknowledgement; HRCP response accepting/recognizing a proposed action.. DENY / MODIFY / WAIT; HRCP responses to reject an action, suggest a change, or intentionally defer execution.. TIMEOUT; Required response not received by deadline; protocol must fall back safely.. E-stop; Emergency stop; independent mechanism to stop hazardous machine motion.. PLC / Safety PLC; Programmable Logic Controller; safety-rated version performs deterministic protective functions independent of AI.. AMR; Autonomous Mobile Robot; mobile robot that navigates and performs tasks autonomously.. CSVR; Critical Safety Violation Rate; critical violations per defined exposure unit/episode count.. p50 / p95 / p99; 50th, 95th and 99th percentiles; used here mainly for decision/control latency tails.. Sim2Sim; Simulation-to-Simulation; validates a policy across different simulators.. Sim2Real; Simulation-to-Real; transfers/validates a simulated policy on physical hardware.. GPU; Graphics Processing Unit; parallel compute used for AI training, inference and simulation.. JSON; JavaScript Object Notation; lightweight structured data format used for HRCP examples/config.. MuJoCo; Multi-Joint dynamics with Contact; physics engine used for robotics/control simulation.. | Term; Meaning. NVIDIA Isaac Lab; GPU-accelerated robotics learning/simulation framework built on Isaac Sim.. GR00T; NVIDIA Project GR00T (Generalist Robot 00 Technology); foundation-model initiative for general-purpose robots.. ASIMOV-Agentic; Google DeepMind robotics-safety evaluation work for unsafe-action refusal, feasibility and escalation.. OWASP; Open Worldwide Application Security Project; publishes practical software/AI security guidance.. ACS; Agent Control Standard; OWASP guidance for runtime controls and governance of agentic AI.. HACTU8; OWASP robotics/IoT/LLM security-testing project; treated here as a project name, not an expanded acronym.. ISO; International Organization for Standardization; publishes robot and machinery safety standards.. ISO/FDIS; ISO Final Draft International Standard; late draft stage before publication.. IEEE; Institute of Electrical and Electronics Engineers; standards/professional body cited for HRI work.. H1 / H2 / H3; Study hypotheses: predictive safety; independent supervisory AI; human–robot coordination.. R0–R4; Illustrative research risk tiers from routine/reversible (R0) to prohibited/unacceptable (R4).. C1–C8; Eight 2×2×2 experiment conditions created by switching MPC, supervisor and HRCP off/on.. G1 / H1 (Unitree); Unitree humanoid robot model names; Unitree H1 is unrelated to this paper’s Hypothesis H1.. |
| --- | --- | --- |

Note: HRCP, its message types and R0–R4 are research constructs proposed in this paper; they are not presented as existing certified standards.